LAB WORK - ELECTROENCEPHALOGRAPHY

Groups of 3 max – 1 acquisition system per group – 1 computer each. SAVE ALL FIGURES

# Materials

Device:

EEG acquisition: MUSE 2 dry electrode EEG system (InterXon ®), 4 channels, wireless (Bluetooth) communication.

N.B. HANDLE WITH CARE!!

Software:

EEG acquisition: Muselsl and LabRecorder

EEG analysis : Matlab and EEGLab

--------------------------------------------------------------------------------------------------------------------------------------

# INSTALLATION

Before playing with the MUSE 2 system, you’ll need to install some python library and dedicated software – This will be done from the Anaconda Terminal

- CREATE A VIRTUAL ENVIRONMENT « muse » :

conda create -n muse
conda activate muse

- INSTALL MUSELSL TO STREAM MUSE 2 DATA

conda install pip

pip install muselsl --proxy http://proxy.isae.fr:3128

conda install -c conda-forge liblsl

- INSTALL THE MUSE DATA VIEWER

pip install vispy

pip install mne

pip install pyqt5

- INSTALL LABRECORDER (TO RECORDE MUSE2 DATA)

https://github.com/labstreaminglayer/App-LabRecorder/releases/tag/v1.16.2

choose LabRecorder-1.16.2-Win_i386.zip

- BUG CORRECTIONS:

Modify backends.py file (C:\Users\Public\Programs\mblock\resources\app\mlink-v2\exec\python-env\win\Lib\asyncio\backends.py)

Change lines 62 et 63 to:

def wrap(gatt_characteristics, data):

value_handle = gatt_characteristics.handle + 1

Modify Stream.py  file (C:\Users\Public\Programs\mblock\resources\app\mlink-v2\exec\python-env\win\Lib\asyncio\streams.py)

Change line 211 to :

outlet.push_sample(data[:, ii])

# INSTALLATION

Open the MUSE 2 box . HANDLE WITH CARE!!

Plug the Bluetooth dongle and turn on the MUSE 2 by pressing the power button for 2 seconds.

The MUSE 2 system has 4 electrodes: TP9, TP10, AF7 and AF8 + auxiliary channel (will not be used today)

Adjust the MUSE 2 system on your forehead and make sur that the rubber electrodes (TP9 and TP10) are clear from any hairs over your hears.

It is important to drop a bit of water with your finger on all the electrodes (including the rubber one TP9 and TP10) – it will ensure optimal signal quality

https://www.youtube.com/shorts/7UOlDKQPu8c?feature=share

Have the setup checked by a supervisor.

--------------------------------------------------------------------------------------------------------------------------------------

# Data streaming

# Start a new Anaconda Terminal and type:

To activate the muse environment, type:

$ conda activate muse

Find the name of your muse device

$ muselsl  stream - -name  name_of_your_device (eg: muselsl stream - - name Muse-D482)

To visualize the data, type:

$ muselsl view - -version 2

the mean values for each electrode should be between 10 and 20 (see figure below)

------------------------------------------------------------------------------------————————————————

# Visualization of artifacts

- While streaming, perform the following activities to generate artifacts on the EEG signal (save screen captures) :

- Blink repeatedly

- Clench your teeth several times

- Move on your chair

- Close you eyes and relax

------------------------------------------------------------------------------------------------—————————————

# A small experiment: Count down vs Relaxation

Evaluation: cognitive workload (on-task vs off-task)

Task: perform a rest session (eyes closed, no movements) followed by a mental count down.

To record your data use Labrecorder (you can find it via Windows search) – please make sure to check the Muse stream and the Marker(MyMarkStream) and then press the Start button

- After the first beep (see console “RELAX”) : 60 seconds of resting state eyes closed  —> Firstname_relax

- After the second beep (see console “Do the Maths”): 60 seconds of the count down session —> Firstname_math

Have the subject perform a mental count down 7 by 7 starting from 700 during one minute. Eyes open & avoid motion!

# Visual Oddball experiment

Evaluation of: attentional orientation

Task: count the rare auditory stimuli

Run…

Blabal…

##### -----------------------------------------------------------------------------------------------------------------

# End of use & cleaning

- Make sure your recorded files are on your computer.

- Stop the recording & streaming.

- Close the software.

- Disconnect the Bluetooth dongle and put it back in its appropriate place in the box.

- Remove the headset and turn the interface off by pressing the button for 2 seconds.

- Clean the electrodes: use a soft cloth and 70 % isopropyl alcohol or disinfection wipes. Clean the pins of all Unicorn Hybrid EEG Electrodes by wiping from different directions over the tips of the pins.

- Put the headset back in its box (with electrodes still on).

- Make sure all equipement is back in the boxes and close them all and put them back in the main box.

--------------------------------------------------------------------------------------------------------------------------------------

# Analyses & Results - using EEGlab

- On the LMS download the folder ‘Practical-EEG’ in which you have the EEGlab toolboxand two P300 files (fdt and set).

- Unzip this folder and put it in your D.

- In Matlab, add the path to the eeglab folder.

- Launch EEGlab: type eeglab and wait for the GUI to appear.

## Rest vs Count down experiment: Spectral analysis

-Load the data of the 1st condition (rest): File, import data, using EEGlab functions and plugins, From Unicorn Hybrid Black

- Select the .csv file of the resting state, then check OK. A new pop-up appears, to rename your file “MyName_rest”.

- Import the channel spherical coordinates: Edit, Dataset info, Channel location file or info

Leave everything as default and click OK

Click on Read locations and select the ‘unicorn_electrodeloc.ced’

Select autodetect to retrieve spherical information based on electrode labels

Correct labels for EEG channels and location coordinates are now imported.

- Now we will discard auxiliary EEG channels: EDIT > Select Data

Click on the box on the right side of the Channel range row. In the pop-up window select the 8 Right auxiliary channels as indicated on the figure. Channel range: click OK on “On->remove these”. Then click “OK” on both windows to close them all.

- Verify the information displayed on the GUI (e.g. number of channel, sampling frequency, number of events).

### Frequency filtering

- Tools, Filter the data, Basic FIR filter, [1 40] Hz, order by default. Rename your file: MyName_ _filtered

- Look at your data: Plot > Channel data (scroll)

### Re-referencin the data (mastoid re-referecing  TP9 and TP10)

- Tools >Re-reference the data. Rename your file: MyName_Relax_Rereferenced

### 3.Spectral analysis

- Plot —> Channel spectra & maps, remove «Frequencies to plot as scalp maps », OK

- Save figure.

### Comparison

- Repeat the whole procedure for the 2nd condition (count down) and save all figures.

- Open the spectra of the 1st and the 2nd condition, adjust the parameters to enable direct comparison (axes).

Remarks on the most appropriate frequency band:

Comparison between the conditions:

————————————————————————————————————————————

## Oddball task: Event-Related Potential analysis

- Load the data: File, import data, oddball_ctrl.set file

- Verify the information displayed on the GUI (e.g. number of channel, sampling frequency, number of events).

- Repeat the same procedure as previously done:

  - Remove auxiliary channel

  - Re-reference the data on TP9 and TP10

### Frequency filtering

- Tools, Filter the data, Basic FIR filter, [1 40] Hz, order by default. Rename your file: Oddball_filtered

- Look at your data: Plot —> Channel data (scroll)

### Epoching

This step allows you to extract epochs of signal synchronized on the markers sent by Matlab and that correspond to the occurrence of a stimulation (100: rare/target sound, 200: standard/distractor sound).

- Tools, extract epochs, in Time-locking event types select the markers 100. Epoch limits: -0.2 1 (seconds), rename your file: Oddball_epochs_100

- Pop-up baseline (sort of normalization): -200 0; rename with same name.

### Artifact rejection

In order to remove noisy  epochs you need to visually inspect the signal and select the epochs to be discarded:

- Plot, Channel data scroll, scroll using the bottom cursor, select the eventual noisy epochs by clicking on the epoch. When all noisy epochs are selected, click on REJECT and rename your file: Oddball_epochs10_cleaned

Perform the whole procedure twice, once for each marker (100 and 200)!

### Event-Related Potentials Visualization:

- Plot, sum/compare ERPs, select the dataset that corresponds to the marker 200 (1st line), then that of the marker 100 (2nd line); select plot difference; plottopo options remove the “-“ sign.

OR type pop_comperp(ALLEEG);

- Click on the graph that corresponds to the Pz electrode.

What do you observe? At which time point is the amplitude maximal?

- Click on the graph that corresponds to the Cz electrode.

What do you observe? Is there a negative peak earlier than the maximal peak you found before?

### Topographical representations

- Plot, Channel ERPs, with scalp maps for each marker type.

What do you observe? Specifically for the max amplitude of the ERP ?